EarthGuard Pad Winterization: Should be specified in situations where bare pads need erosion protection. Mulch can be added for additional protection, but it is not required.

THE SPECIAL PROVISIONS DESCRIBED WITHIN IS AN ATTEMPT TO COVER MOST DESIGN ASPECTS OF UTILIZING THE EARTHGUARD FOR PAD WINTERIZATION. THIS DOCUMENT IS INTENDED TO BE UTILIZED AS A GUIDELINE, WHICH CAN BE CUSTOMIZED BY THE USER TO MEET SPECIFIC PROJECT REQUIREMENTS.

THIS PROVISION DOES NOT ATTEMPT TO ADDRESS ALL ASPECTS OF PAD WINTERIZATION. A PROFESSIONAL ENGINEER RESPONSIBLE FOR DESIGN SHOULD CHECK THE INFORMATION PRESENTED IN DETAIL. CONSULT YOUR EARTHGUARD REPRESENTATIVE FOR MORE INFORMATION.

SOME SITUATIONS WITH: SANDY-SANDY LOAM SOILS, VERY LITTLE ORGANIC MATTER IN THE SOIL, POORLY COMPACTED SLOPES, HEAVY RAINFALL LOCATIONS (OVER 20 INCHES PER YEAR), VERY INTENSE RAIN STORMS (2-3 INCHES AT A TIME), HIGH WIND EXPOSURE, ETC. MAY REQUIRE INCREASING THE EARTHGUARD. WHEN SWPPPS, BMPS OR OTHER PLANS ARE BEING WRITTEN, CONTACT YOUR EARTHGUARD REPRESENTATIVE AT (888) 843-1029 FOR MORE SPECIFIC RATE RECOMMENDATIONS BASED ON SOIL TYPE, RAINFALL, SLOPE AND OTHER CONDITIONS.

1.  GENERAL

1.1.  Work Included.  EarthGuard Pad Winterization shall be furnished and applied as shown on the drawings and as required for proper installation and functioning of the product.

2.  MATERIALS

2.1. Liquid Chemical Blend. The liquid chemical blend to be applied shall be “EarthGuard” as manufactured by Terra Novo Inc (888) 843-1029. EarthGuard can be stored for 6 months. Always thoroughly mix before using. Always close or seal containers after use. Store in a dry, insulated location to avoid moisture and freezing. If frozen, thaw and completely remix the solution.

- EARTHGUARD PAD WINTERIZATION

EarthGuard Pad Winterization shall consist of spraying “EarthGuard” with enough water to sufficiently moisten the area to be covered. Consult your EarthGuard Sales Representative and/or product manufacturer Terra Novo Inc at (888) 843-1029 for the appropriate project specific application rates.

| TIME (months) | EARTHGUARD+ (gal/ac) | WATER (gal/ac) |
| --- | --- | --- |
| 1-2 | 3-5 | AS REQUIRED TO COVER TO COVER 1 AC OF AREA AND DEPENDENT ON EQUIPMENT’S ABILITY TO PROPERLY APPLY Hydroseeder ≈ 2000 gal/ac Spray Rig ≈ 2000 gal/ac Water Truck ≈ 2500 gal/ac |
| 2-3 | 5-8 |  |
| 3-6 | 8-10 |  |
| 6-12 | 10-15 |  |
| + Application Rates dependent on anticipated precipitation throughout duration of required protection. |  |  |

- BLENDING

Prior to blending, thoroughly mix the EarthGuard in its container. The mixing vessel shall be filled with water to at least 1/3 capacity prior to adding any EarthGuard. As the tank continues to fill slowly add the EarthGuard into incoming water stream. If possible, mix and circulate tank for a minimum of 10 minutes prior to application.

- APPLICATION

EarthGuard Pad Winterization shall be applied evenly over the entire area to be treated. Caution should be taken to avoid creating puddles or runoff. Treated areas shall not be disturbed after application.

Correct mixing and proper application are key factors in obtaining satisfactory performance.

A Certificate of Compliance shall be furnished to the Engineer stating that the appropriate quantities, blending and application methods as specified here within have been properly followed.

6. REPAIR OF DAMAGED AREA

Any damaged area shall be repaired utilizing the exact blend and application procedure as specified above.

- PAYMENT

- EarthGuard Pad Winterization shall be paid for by the square foot of area treated. The price shall include full compensation furnishing all labor, materials, tools, equipment, and incidentals, and for doing all Pad Winterization work, complete in place, as shown on the plans, and as specified in these special provisions, and as directed by the Engineer.

EarthGuard Document of Compliance

Applicator:____________________________		Date:_______________________

Tract/Proj. No.:________________________		Proj. Manager:_______________

Developer/Owner:______________________		Proj. Location:________________

Proj. Name:___________________________		Proj. City:___________________

| # SIZE (acres) |  |
| --- | --- |
| EarthGuard (gal/acre) |  |

I, ____________________________________(name)_____________________________________(applicator company)

hereby declare that the information here within is true and correct to the best of my knowledge and belief. I further declare the above listed rates have been applied as specified by the contractor, developer, plans, and specifications or as directed by an owner representative on the site. I further declare the above listed products have been applied in a manner consistent with Terra Novo's recommended application procedures and methods. Any deviation from the plans, specifications, direction by the owner representative, or Terra Novo's recommended application procedures and methods are listed below.

Signature:______________________________	Date:_____________________

Deviations:__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ _________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________