### SPECIAL PROVISIONS

EarthGuard a Rain Event Action Plan (REAP) BMP:

Use these specifications where a construction site is under active construction and there is an impending storm event requiring soil stabilization to help stay in compliance with the California General Construction Permit. This is a water truck or spray rig application.

Construction can continue unencumbered after a storm.

### SPECIAL PROVISIONS

EarthGuard a Rain Event Action Plan (REAP) BMP:

THE SPECIAL PROVISIONS DESCRIBED WITHIN IS AN ATTEMPT TO COVER MOST DESIGN ASPECTS OF UTILIZING THE EARTHGUARD AS A RAIN EVENT ACTION PLAN (REAP) BMP FOR EROSION PROTECTION. THIS DOCUMENT IS INTENDED TO BE UTILIZED AS A GUIDELINE, WHICH CAN BE CUSTOMIZED BY THE USER TO MEET SITE-SPECIFIC PROJECT REQUIREMENTS.

THIS PROVISION DOES NOT ATTEMPT TO ADDRESS ALL ASPECTS OF EROSION PROTECTION. A PROFESSIONAL ENGINEER RESPONSIBLE FOR DESIGN SHOULD CHECK THE INFORMATION PRESENTED IN DETAIL. CONSULT YOUR EARTHGUARD REPRESENTATIVE FOR MORE INFORMATION.

SOME SITUATIONS WITH: SANDY-SANDY LOAM SOILS, VERY LITTLE ORGANIC MATTER IN THE SOIL, POORLY COMPACTED SLOPES, HEAVY RAINFALL LOCATIONS, VERY INTENSE RAIN STORMS (2-3 INCHES AT A TIME), HIGH WIND EXPOSURE, ETC. MAY REQUIRE INCREASING THE EARTHGUARD BY 1.25-1.50 TIMES.

WHEN SWPPPS OR EROSION CONTROL PLANS ARE BEING WRITTEN, CONTACT YOUR EARTHGUARD REPRESENTATIVE AT (888) 843-1029 FOR MORE SITE-SPECIFIC RATE RECOMMENDATIONS BASED ON SOIL TYPE, RAINFALL, SLOPE AND OTHER CONDITIONS.

### SPECIAL PROVISIONS

EarthGuard a Rain Event Action Plan (REAP) BMP:

1.  GENERAL

1.1.  Work Included.  EarthGuard for Rain Event Action Plan Protection shall be furnished and applied as shown on the drawings and as required for proper installation and functioning of the product.

2.  MATERIALS

2.1. Liquid Chemical Blend. The liquid chemical blend to be applied shall be “EarthGuard” as manufactured by Terra Novo Inc (888) 843-1029. Always thoroughly mix before using. Always close or seal containers after use. Store in a dry, insulated location to avoid moisture and freezing. If frozen, thaw and completely remix the solution.

- EARTHGUARD a RAIN EVENT ACTION PLAN BMP

Rain Event Action Plan protection shall consist of a combination of “EarthGuard” and water mixed in the following proportions. Consult your EarthGuard Sales Representative and/or product manufacturer Terra Novo Inc at (888) 843-1029 for the appropriate project specific application rates.

| SLOPE | ≤ 4:1 | 3:1 | 2:1 | 1.5:1 | 1:1 | Stockpiles |
| --- | --- | --- | --- | --- | --- | --- |
| EARTHGUARD (gal/ac) | 3 | 4 | 5 | 6 | 8 | 10 |

- BLENDING

Prior to blending, thoroughly mix the EarthGuard in its container. The mixing vessel shall be filled with water to at least 1/3 capacity (high enough to cover agitators) prior to adding any EarthGuard. As the tank continues to fill, slowly add the EarthGuard directly to the fill water flow.

- APPLICATION

EarthGuard a REAP BMP shall be applied evenly over the entire area to be treated. Caution should be taken to avoid creating runoff or puddles. Correct mixing and proper application are key factors in obtaining satisfactory performance.

A Certificate of Compliance shall be furnished to the Engineer stating that the appropriate quantities, blending and application methods as specified herewithin have been properly followed.

- REPAIR OF DAMAGED AREA

Any damaged area shall be repaired utilizing the exact blend and application procedure as specified above.

- PAYMENT

- EarthGuard a REAP BMP shall be paid for by the square foot of area treated. The price shall include full compensation furnishing all labor, materials, tools, equipment, and incidentals, and for doing all EarthGuard a REAP BMP work, complete in place, as shown on the plans, and as specified in these special provisions, and as directed by the Engineer.

EarthGuard REAP BMP Document of Compliance

Applicator:____________________________		Date:_______________________

Tract/Proj. No.:________________________		Proj. Manager:_______________

Developer/Owner:______________________		Proj. Location:________________

Proj. Name:___________________________		Proj. City:___________________

| # SLOPE | ≤ 4:1 | 3:1 | 2:1 | 1½: 1 | 1:1 |
| --- | --- | --- | --- | --- | --- |
| # SIZE |  |  |  |  |  |
| (acres) |  |  |  |  |  |
| EarthGuard |  |  |  |  |  |
| (gal/acre) |  |  |  |  |  |

I, ____________________________________(name)_____________________________________(applicator company) hereby declare that the information herewithin is true and correct to the best of my knowledge and belief. I further declare the above listed rates have been applied as specified by the contractor, developer, plans, and specifications or as directed by an owner representative on the site. I further declare the above listed products have been applied in a manner consistent with Terra Novo's recommended application procedures and methods. Any deviation from the plans, specifications, direction by the owner representative, or Terra Novo's recommended application procedures and methods are listed below.

Signature:______________________________	Date:_____________________

Deviations:_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________ __________________________________________________________________________________________________________________________________________